Pressure Sensing Mat

PCB Design Specification

Rev 2.0  |  March 2026  |  USB-Powered

# 1. System Overview

This document specifies a pressure-sensing mat system consisting of three PCBs:

- Two flexible PCBs (row layer + column layer) that sandwich a Velostat piezoresistive film to form a pressure-sensing matrix

- One rigid main board containing an ESP32-S3 microcontroller, analog multiplexers, and pull-down resistor networks

The system is USB-powered and communicates with a host computer over USB serial (CDC). There is no battery and no wireless radio required.

## 1.1 Physical Form Factor

The sensing mat sits on a flat surface approximately 30 cm wide x 45 cm long. The mat has two independent sensing zones (left and right), each 12 rows x 28 columns = 336 sensing points. Total matrix: 12 rows x 56 columns = 672 points.

The main electronics board connects to the flex PCBs via two FFC/FPC ribbon cables (one per layer). The main board sits off the mat and is housed in a small enclosure.

# 2. Sensor Mat Assembly

## 2.1 Layer Stack-Up (Bottom to Top)

| Layer | Material | Description |
| --- | --- | --- |
| 1 | Bottom flex PCB (Row layer) | 12 horizontal copper traces, 2.0 mm pitch, bare copper pads exposed on top surface. FPC connector tail on one edge. |
| 2 | Velostat film | 3M Velostat 1361, single sheet cut to mat dimensions. Placed directly on top of row layer. |
| 3 | Top flex PCB (Column layer) | 56 vertical copper traces (28 per zone), 5.0 mm pitch, bare copper pads exposed on bottom surface facing the Velostat. FPC connector tail on one edge. |
| 4 | Silicone cover | Protective top layer, 1-2 mm silicone or rubber sheet. Not a PCB deliverable. |

## 2.2 Flex PCB Specifications

| Parameter | Row Layer (Bottom) | Column Layer (Top) |
| --- | --- | --- |
| Trace count | 12 | 56 (28 left + 28 right) |
| Trace pitch | 2.0 mm center-to-center | 5.0 mm center-to-center |
| Trace width | 1.2 mm | 3.5 mm |
| Pad geometry | Full-width strips running across mat | Full-length strips running down mat |
| Surface finish | Bare copper (no solder mask) on sensing side; solder mask on back | Bare copper (no solder mask) on sensing side; solder mask on back |
| Substrate | Polyimide (Kapton), 0.1 mm | Polyimide (Kapton), 0.1 mm |
| Copper weight | 1 oz (35 µm) | 1 oz (35 µm) |
| Connector | FPC tail, 0.5 mm pitch, 12-pin | FPC tail, 0.5 mm pitch, 56-pin |
| Overall dimensions | ~300 mm x 450 mm + connector tail | ~300 mm x 450 mm + connector tail |

Note: The sensing side of each flex PCB must have NO solder mask or coverlay over the copper pads. The pads must make direct contact with the Velostat film.

# 3. Main Electronics Board

## 3.1 Block Diagram

USB-C (5V power + data) → ESP32-S3-DevKitC → GPIO drives row multiplexer → one row HIGH at a time

Column traces → pull-down resistor network (10kΩ each) → column multiplexers → ESP32 ADC input

ESP32 reads all 56 columns per row, cycles through 12 rows = 672 readings per scan frame. Data sent to host via USB CDC serial.

## 3.2 Scanning Circuit

Each sensing point is a voltage divider. The Velostat resistance between a row trace and column trace varies with applied pressure (high resistance = no load, low resistance = load). The circuit for each column:

Row drive (3.3V when selected) → Velostat resistance (Rv) → Column trace → 10kΩ pull-down to GND

V_col = 3.3 × (10k / (Rv + 10k))

At no load, Rv ~ 50-100kΩ, V_col ~ 0.3-0.6V. At full load, Rv ~ 1-5kΩ, V_col ~ 2.2-3.0V. This maps well to the ESP32 12-bit ADC range (0-3.3V).

## 3.3 Row Driving

12 rows are driven one at a time. Use two CD74HC4067 16:1 analog multiplexers (only one needed for 12 rows, second is spare or use a single 74HC4051 8:1 + 4 direct GPIO). The selected row is driven HIGH (3.3V), all others are tristated or LOW.

Alternative: drive rows directly from 12 GPIO pins (simpler, uses more pins but ESP32-S3 has plenty).

Note: Preferred approach: direct GPIO drive for rows (12 pins). This avoids a mux for driving and simplifies the layout. Use muxes only on the column (reading) side.

## 3.4 Column Reading

56 columns must be read through the ESP32 ADC. The ESP32-S3 has 2 ADC units but limited pins. Use analog multiplexers:

- Option A: 4x CD74HC4067 (16:1 mux). Each handles 14 columns. 4 mux output lines → 4 ADC pins. 4 shared address lines (S0-S3) select which of 16 inputs to read.

- Option B: 7x CD74HC4051 (8:1 mux). Each handles 8 columns. 7 mux output lines → 7 ADC pins. 3 shared address lines (A/B/C).

Note: Recommended: Option A (4x CD74HC4067). Fewer ICs, fewer ADC pins consumed, simpler routing.

## 3.5 Pull-Down Resistor Network

Each of the 56 column traces needs a 10kΩ pull-down resistor to GND. Use resistor arrays (SIP or SMD network) to save board space:

- 7x 8-element isolated resistor arrays (10kΩ), e.g., Bourns 4609X-101-103LF (8 resistors, 9 pins, SIP package), or

- 14x 4-element 0603 resistor arrays, or 56 individual 0603 10kΩ resistors

# 4. Bill of Materials (Main Board)

| # | Component | Part Number | Package | Qty | Unit $ | Ext $ |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ESP32-S3-DevKitC-1 (N16R8) | ESP32-S3-DevKitC-1-N16R8 | Dev board | 1 | $8.00 | $8.00 |
| 2 | 16:1 Analog Mux | CD74HC4067M96 | SOIC-24 | 4 | $0.80 | $3.20 |
| 3 | 10kΩ Resistor Array (8-element) | 4609X-101-103LF | SIP-9 | 7 | $0.30 | $2.10 |
| 4 | FPC Connector 12-pin (row layer) | Molex 5051101291 or equiv | 0.5mm pitch SMD | 1 | $0.50 | $0.50 |
| 5 | FPC Connector 56-pin (col layer) | Molex or Hirose 56-pin FPC | 0.5mm pitch SMD | 1 | $1.50 | $1.50 |
| 6 | Decoupling caps 100nF | Generic MLCC | 0603 | 6 | $0.02 | $0.12 |
| 7 | Pin headers (for ESP32 dev board) | Standard 2.54mm | 2x20 female | 2 | $0.40 | $0.80 |
| 8 | Velostat sheet (3M 1361) | 3M Velostat 1361 | Sheet, cut to size | 1 | $5.00 | $5.00 |
|  |  |  |  |  | Total | ~$21 |

Estimated total unit cost: ~$21 (excluding flex PCB fabrication). Flex PCB fabrication typically $15-30 per set at prototype quantities.

# 5. ESP32-S3 Pin Assignment

| GPIO | Function | Notes |
| --- | --- | --- |
| GPIO 1-4 | Mux address S0-S3 | Shared across all 4 column muxes. Select which of 16 inputs to read. |
| GPIO 5-8 | ADC inputs (mux out) | One per column mux. Must be ADC1-capable pins (not ADC2). |
| GPIO 9-20 | Row drive (12 pins) | Each drives one row trace HIGH/LOW. Direct GPIO, no mux needed. |
| GPIO 19-20 (USB) | USB D- / D+ | Native USB on ESP32-S3. Used for CDC serial data and 5V power input. |

Note: GPIO 19/20 are the ESP32-S3 native USB pins. The DevKitC board routes these to the USB-C connector. USB provides both 5V power and CDC serial data link. No additional UART or power circuitry needed.

# 6. Scanning Timing Budget

Target: 50 Hz full-frame scan rate (20 ms per frame).

Per frame: 12 rows x 56 columns = 672 ADC readings.

Available time per reading: 20,000 µs / 672 = ~29.8 µs.

ESP32-S3 ADC conversion: ~10-15 µs per sample at 12-bit. Mux settling time: ~1 µs. Total per reading: ~16 µs. This leaves comfortable margin for row switching overhead.

Note: 50 Hz is the minimum scan rate. The hardware should support up to 100 Hz if needed (still within timing budget at ~15 µs per reading).

# 7. USB Interface

## 7.1 Connection

The ESP32-S3 DevKitC has a built-in USB-C connector connected to the native USB peripheral (GPIO 19/20). This single USB-C connection provides:

- 5V power to the entire system (ESP32 onboard LDO regulates to 3.3V)

- CDC serial data at 921600 baud (virtual COM port, no FTDI chip needed)

## 7.2 Data Output Format

Each scan frame is sent as a binary packet over USB serial:

| Field | Size | Description |
| --- | --- | --- |
| Sync header | 2 bytes | 0xAA 0x55 (frame start marker) |
| Frame counter | 2 bytes | uint16, wrapping counter |
| Timestamp | 4 bytes | uint32, microseconds since boot |
| Matrix data | 672 x 2 bytes | 12-bit ADC values, row-major order (row 0 col 0..55, row 1 col 0..55, ...) |
| Checksum | 2 bytes | CRC-16 over all preceding bytes |

Total packet size: 2 + 2 + 4 + 1344 + 2 = 1,354 bytes per frame.

At 50 Hz: 67,700 bytes/sec. USB CDC serial at 921600 baud handles this easily.

# 8. Design Considerations

- Analog ground plane: keep analog mux traces and ADC input traces away from digital noise (row drive switching). Use a solid ground plane under the analog section.

- Decoupling: 100nF cap on VCC of each mux IC, close to the pin. One bulk 10µF near ESP32 VCC.

- FPC connector placement: position both connectors on the same edge of the main board for clean cable routing.

- Mux enable pins: tie all mux EN pins to GND (always enabled) or route to a spare GPIO if power gating is desired later.

- ADC reference: ESP32-S3 ADC uses internal reference. No external VREF needed.

- Board size target: ~50 mm x 70 mm (roughly credit-card size). The ESP32 DevKitC mounts via pin headers.

- Mounting: include 4x M3 mounting holes at corners.

# 9. Requested Deliverables

- Flex PCB layout: Row layer (12-trace, 0.5mm pitch FPC tail, 12-pin). Gerber files + fabrication notes.

- Flex PCB layout: Column layer (56-trace, 0.5mm pitch FPC tail, 56-pin). Gerber files + fabrication notes.

- Main board schematic (KiCad or Altium). ESP32-S3-DevKitC as module on pin headers, 4x CD74HC4067, resistor arrays, FPC connectors.

- Main board PCB layout. Gerber files, BOM, pick-and-place file.

- Schematic review: confirm mux addressing logic, ADC pin assignments, and pull-down values.

- DRC clean: no unconnected nets, clearance violations, or DRM errors.